Acme Washing Ozone Laundry Monitoring System Proposal

Prepared by Daniel AI

### Overview

Acme Washing is a laundry supplier that uses an innovative low-temperature, ozone-based washing system. This provides similar or better cleaning performance to traditional high-temperature laundry solutions, but with the advantage of being greener and having lower operating costs.

As part of this solution, Acme Washing supplies a range of ozone laundry system machines which are designed to be integrated into the water feed to an existing industrial washing machine.

### Development Brief

At present, it is not possible to monitor the ozone machines remotely in any way. Acme Washing is seeking to extend the existing system such that it is possible to monitor various parameters of a unit’s performance remotely, and provide alerts when faults occur.

Acme Washing have highlighted that they would ideally like the ability to remotely monitor the following:

- Water flow

- Air flow

- Current flow

- Faults raised by other parts of the system such as the PLC

Each individual machine should be able to be monitored remotely through a web-based front-end. In addition, the system should have the ability to raise an alert by email, should one of the monitored parameters go out of the expected operating range.

### Evaluation of the Existing System

The ozone machine comprises of a large cabinet with two sections that are kept separate from each other. The electrical panel side comprises various sensors, fuses and contacts, a programmable logic controller (PLC), the power supply and a pair of oxygen concentrators. The ‘water side’ contains the pipework for the incoming and outgoing water supply where the ozone is introduced.

On the exterior of the unit are switches for controlling the power, pumps and ozone output, and status LEDs showing ozone operation, pump operation, power and an LED to show if a fault has occurred.

At present, if a fault occurs in the system, the only obvious indication of this is the lighting of the fault LED on the exterior of the machine. If the fault LED lights, it is then up to a qualified operator to open the machine to inspect the display of the PLC for further diagnostic information.

### Hardware Suggestions

The following are our suggestions for the hardware that might be suitable for this project. Please not that these are not recommendations – any suggestions would need to be scrutinised by properly qualified electrical and electronic engineers.

Microcontroller

To monitor the various parameters of the ozone machine, there will need to be some kind of microcontroller or single-board computer to manage collection of the data. We initially considered use of a Raspberry Pi single-board computer, but this is unsuitable for industrial applications as it may not be able to react quickly to sensor changes and is prone to interference and unreliability in an industrial setting. As a result, we propose building a monitoring system around an Arduino-based microcontroller. We have identified that an Arduino GIGA R1 Wi-Fi would potentially be suitable for this system. This is a small single-board microcontroller that is fully programmable in a high-level language and is ideal for reading multiple sensors in real time. https://uk.rs-online.com/web/p/arduino/2623452

We propose to develop software for the Arduino which would collate the data from the various sensors and then upload it to a remotely hosted web service over the Arduino’s built-in WiFi connection. Please see the Connectivity section below for further details of how this could be extended to include connectivity over a mobile signal and/or an ethernet network connection.

Water Flow Monitoring

Several options are available for the monitoring of water flow.

Firstly, if the requirement is simply a record of when water is flowing and when it is not, then the simplest and most cost-effective solution is simply to wire the output that goes to the water pump LED on the ozone machine to a digital input on the Arduino also. This would allow us to report to the web service when the water pump is activated, but it will not provide any indication of water flow rates.

If full flow rate measurement is required, then there are two potential options. The first is to fit a full-size flow rate sensor inline with the water output from the machine. This would be a full 2” unit capable of handling up to the 250l/min limit of the equipment. This would provide a pulse to the Arduino for every 10 litres of water moved, allowing calculation and recording of the flow rate. A suitable water flow meter would be this unit: https://www.bes.co.uk/pulsed-cold-water-meter-2-23993

The disadvantage of this approach is that flow meters at this size are somewhat expensive due to the required flow rate , and it may not be cost effective to pursue this route.

Another potential option is to split off a small part of the water flow into a smaller diameter pipe that can then be measured with a smaller water flow sensor (e.g. something that can measure up to 30 litres / minute). This would allow an approximation of the full flow to be calculated. The disadvantage of this is that it potentially introduces a new point of failure in the system.

Smaller flow sensors are much more cost effective. A suitable sensor might be the following: https://uk.farnell.com/dfrobot/sen0217/water-flow-sensor-arduino-board/dp/3517923

Ozone Flow Monitoring

Several options are available for monitoring ozone flow.

As with water flow, if the requirement is simply a record of when ozone is flowing and when it is not, then the simplest solution is to wire the connection going to the ozone LED indicator on the ozone machine to the Arduino also. This would allow us to report to the web service when the ozone pump is activated, but it will not provide any indication of ozone flow rates.

If actual flow rate measurement is required, then mass air flow sensors that will interface with an Arduino are available, however they are relatively expensive. The following air flow sensor can handle up to 150 litres/minute airflow and would be suitable for connection to an Arduino. https://www.digikey.co.uk/en/products/detail/dfrobot/SEN0360/13166496

Current Monitoring

Current monitoring is relatively straightforward and can be achieved without any direct connection to the electrics of the unit. The following AC current sensor clamps around the AC power supply cable and is supplied with a small interface unit which converts the signals to a form suitable for an Arduino. It is suitable for up to 20A, so is suitable for the 16A supply used by the ozone machine. https://uk.farnell.com/dfrobot/sen0211/analogue-ac-current-sensor-arduino/dp/3517919

Fault Monitoring

We have investigated the possibility of interfacing directly with the unit’s own programmable logic controller to determine faults that have been detected. This would only be possible if the software on the PLC can place the fault signals onto the output relays of the controller. Failing this, it would be possible to interface the signal going to the ozone unit’s fault LED to the Arduino also, so that it could report on the overall fault condition of the machine itself.

Signal Conversion

Signals that go to the ozone machine’s external LEDs run at 24V DC. The Arduino unit expects digital inputs to be in the range of 3.3 – 5V DC, and therefore it will almost certainly be necessary to convert the signals to a form suitable for the Arduino microcontroller. This can be done using an optocoupler to convert the signals down to 3.3V and provide the necessary isolation to not destroy the electronics of the Arduino. These are relatively inexpensive and a single optocoupler unit can handle multiple signals at once. https://www.botnroll.com/en/converters-and-isolators/4558-24v-to-3v3-8-channel-optocoupler-isolation-module-pnp-output-plc-signal-converter.html

Connectivity

Out of the box, the Arduino microcontroller only supports WiFi connectivity. This may or may not be ideal due to the industrial setting that the machines are likely to be placed in. As a result, it is possible to provide alternative connectivity, potentially as an add-on module should a particular customer require it.

Potential connectivity options include the following:

Ethernet Networking: It is possible to add an ‘ethernet shield’ to the Arduino unit, that provides ethernet connectivity. This would then allow the unit to be connected to the customer’s own local area network. https://uk.rs-online.com/web/p/shields-for-arduino/8732285

4G/5G Mobile Data: Connectivity over mobile data could also be provided by way of a mobile hotspot fitted within the machine itself. This would have a SIM card inserted and would present itself as a WiFi network to which the Arduino could connect. The following unit is intended for industrial applications and as a result is ideal for this purpose. https://uk.rs-online.com/web/p/routers/0428077

This unit would almost certainly also require an external antenna, as the case of the ozone machine is likely to block mobile signals otherwise. A suitable antenna might be the following: https://uk.rs-online.com/web/p/multi-band-antennas/2024093

### Reporting/Alerting

We propose that the Arduino would upload the data it collects to a remotely hosted web service every 15 minutes. This web service would have a web-based front end, which would enable the user to view data such as the current and historical statuses of a particular machine. The system would also allow the user to specify the expected parameters for each machine and who to alert if the system exceeded those parameters.

This service could be hosted either on an existing Acme Washing server if one were available, or with a suitable cloud provider such as Amazon AWS. We would build this part of the system using a combination of Vue for the front-end and Microsoft .NET for the back end. Access would be via username/password authentication and could be extended to include two-factor authentication if required.

Should the system need to be hosted with a cloud provider then there would be an ongoing cost implication for this. We would estimate that this would be in the range of £50-£75 per month.

### Assumptions Made

We have made sensor recommendations based on our best knowledge of the existing systems. However, as we are software engineers and not hardware engineers, these will need to be checked in more detail for suitability, particularly with regards to flow ratings and electrical compatibility with the Arduino.

It is assumed that there will need to be some sort of prototyping phase, and that the final hardware design may need to evolve based on the results of initial prototyping.

We have also assumed that installation of the hardware will be carried out by Acme Washing themselves.

### Outline Task List and Costs

We have split the development into tasks and established rough estimates of time for each.

If the requirements were to change mid-project, or if unexpected issues are discovered, the chargeable time may go up or down. Acme Washing would only be billed for the actual time spent, as documented with a breakdown of timesheet data.

| Task | Days |
| --- | --- |
| Microcontroller |  |
| WiFi/4G provisioning routines | 3 |
| Sensor data capture routines | 4 |
| Web service data upload routines | 3 |
| Web Service Back End |  |
| Data access layer | 0.25 |
| Authentication routines | 0.5 |
| Receive data from microcontroller | 0.5 |
| Data processing routines | 2 |
| Alerting/email routines | 0.5 |
| Configuration routines | 0.5 |
| Web Service Front End |  |
| Basic site design | 1 |
| Configuration page | 0.5 |
| Dashboard pages | 3 |
| Misc |  |
| Testing and rework | 2 |
| Web service setup and deployment | 1 |
| Total Days | 21.75 |
| Total Cost @ £75 / hr (excluding VAT) | £12,234.38 |

Rectangle Red operates with a transparent timesheet system which provides details of the tasks worked on each period, and how much time was spent on each. Our daily rate is £75/hour, plus VAT, and this is charged on a pro-rata basis, so that, for example, if a half hour’s work is needed on a given day, only that half hour is chargeable. No retainer fee is charged; therefore, if no work is carried out in a period, no fee is payable. Invoices are sent in arrears at the end of each calendar month for work carried out in the previous month.

Once paid for, any intellectual property in the work is fully owned by Acme Washing, and not Rectangle Red.

### Summary and Next Steps

We have established a likely budget for this task of £12,234.38 + VAT to cover 21.75 days of work. If you are happy to proceed, we can provide you with a consultancy agreement which fixes the development rate and ensures you have the ownership of the intellectual property rights. Once signed by both parties, we will be able to commence work.

During development, we will give you project updates and the opportunity to view work-in-progress, so that any issues can be identified during the development process and corrected easily.

Thank you for the opportunity to discuss the project with you - we would very much like the opportunity to work with you and hope to be able to build the contractor certification portal to meet all your expectations.

[[[ END OF DOCUMENT ]]]